In this demonstration we will look at Nesting in ArtCAM Insignia. ArtCAM Insignia has a true shape nesting library that allows irregular shapes to fit together as closely as possible.

# True Shape Nesting

- Click on Open Model and select Nested dino.art.

- Using the left mouse button, drag a large box around all of the artwork to select all the vectors.

- Select the Nesting function from the Vector Creation toolbar.

- With the Nesting panel open use the following values:

- Tool diameter (D) - 12mm

- Toolpath Clearance - 12mm

- Sheet Edge Clearance - 20mm

- Check the  Allow Part Rotation

- Step Angle (A):  45

- Tick  Model is Sheet

- Tick  Group Nested Vectors

- Click  Nest

- Click on the X to close the Nesting panel

- Now that the shapes are nested together the next step is to apply a toolpath to the piece.

- On the Project panel, click on Toolpaths to show the toolpath creation functions.

- Using the left mouse button, drag a box around all of the vectors to select them all.

- Click on the Profile toolpath function located in the lower half of the panel.

- The Profiling function appears automatically:

- Use the following values:

- Profile option to Outside.

- Offset Allowance -  0

- Start Depth - 0

- Finish Depth -  18

- Tolerance – 0.001

- Click on the Click to Select... to select the Profiling Tool

- The Tool Database appears, select the 6mm End Mill from the Metric Tools - Wood or Plastic – Roughing and 2D Finishing  section

- Material Thickness Setup:

- Material Thickness - 18mm

- Material Z Zero – Check top box

- Model Position In Material – Check top box

- OK

- Click Calculate Now

- Click on the X to close the toolpath panel

- The next step is to add tabs/bridges onto the vectors so that the pieces do not fall out of the material after they have been cut out on the machine.

- In the project Panel, click on the plus sign next to the calculated Profile toolpath

- Select Profile toolpath End Mill 6mm

- Click the Profile Options function available at the bottom half of the Project Panel

- The Profile options gets open in the Tools settings: Panel

- Choose Bridges as the option and use the following values:

- Add Bridges To Profiles – Constant Number

- Number – 2

- Bridge Length – 12

- Bridge Thickness –  6

- Make sure 3D Bridges is Unchecked

- Click Create Bridges

- We will now move the bridges to more sensible positions which will be easier to remove it.

- On the 2D View drag the bridges around manually till they are on a straighter section. This can be done by moving the mouse over the centre of each bridge, so that the cursor changes to a circle with a cross going through, Hold the Left mouse button down, and drag the Bridge to straighter section.

- Carry this procedure until all the vectors has the bridges applied on the straight section

- Press Esc on the keyboard to exit the Profiling options function

- Click on the 3D View Tab to view the toolpath in 3D View with the Bridges applied to it.

- To simulate the piece right click on the Profile toolpath in the Project panel and select Simulate Toolpath.

- ArtCAM Insignia gives you the option to change the resolution of the simulation so you can view the finished piece more clearly, ArtCAM Express has a fixed resolution.

- In ArtCAM Insignia select Simulate Toolpath (ArtCAM Express will do this automatically).

- Select Toolpaths section from the Project panel to view all the toolpath creation functions

- From the Toolpath Operations section click on the Save Toolpaths              function located in the lower half of the project panel to open the Save Toolpaths box.

- From the Machine output file is formatted for: box select your machine from the pulldown and select Save.

- In the File Name: section type in Dinosaur and select Save to save your toolpath ready to load onto your machine.